# Machine Repair & Troubleshooting Knowledge Guide

Machine ID: 00WH9OUI | Manufacturing Agentic AI | IIIoT Infotech

Purpose: This document is designed for RAG retrieval by a Machine Investigation Agent. It provides symptom-based triage, sensor validation, inspection guidance, escalation rules, and verification steps. It is not a substitute for OEM manuals, certified electrical procedures, or an authorized maintenance engineer.

# 1. Machine Telemetry Parameters

| Parameter Group | Signals | Unit | Agent Use |
| --- | --- | --- | --- |
| Voltage | BN_V, BR_V, RN_V, RY_V, YB_V, YN_V, V_AVG | V | Detect undervoltage, overvoltage, phase imbalance, and invalid readings |
| Current | B_Current, R_Current, Y_Current, Current_AVG | A | Detect overload, phase current imbalance, and abnormal demand |
| Thermal | Temperature | °C | Detect overheating, sensor faults, and implausible readings |
| Machine State | Dry_Status | status | Check dry-run or process-state condition according to OEM definition |
| Production | Cumulative_Cycles, Proxy_Count | cycles/count | Track operating activity and maintenance context |

# 2. Important Validation Rules

Invalid temperature: IF Temperature is below a physically plausible plant-defined minimum or equals a known sensor fallback value (for example, -129.6°C), THEN mark the signal as SENSOR_INVALID, do not treat it as a healthy temperature, inspect sensor wiring/scaling, and compare with an independent thermometer or PLC value.

Impossible or inconsistent voltage: IF a voltage reading is outside the configured operating range or inconsistent with related phase-to-neutral/phase-to-phase readings, THEN classify as VOLTAGE_ABNORMAL, verify the measurement point, wiring, scaling, and meter reading before any repair.

Zero or missing signal: IF a required signal is zero, null, stale, or unchanged beyond the configured freshness window, THEN classify as DATA_QUALITY_ISSUE and verify sensor, gateway, PLC, InfluxDB timestamp, and connectivity.

Conflicting status: IF the UI says normal while a value violates the configured threshold, THEN the agent must prioritize the validated numeric rule, flag a threshold/UI synchronization issue, and request review of threshold configuration.

# 3. Troubleshooting Decision Rules

| Condition / IF | Possible Causes | THEN: Immediate Action | Verification | Escalation |
| --- | --- | --- | --- | --- |
| IF Average Current or any phase current exceeds the approved operating limit | Overload, mechanical friction, bearing issue, phase imbalance, supply problem, incorrect threshold, or sensor error | Stop or isolate only according to approved site procedure; notify maintenance/electrical personnel; do not repeatedly restart | Check phase currents, motor load, mechanical movement, nameplate rating, and calibrated meter | Escalate if current remains high, protection trips, smoke/heat is present, or imbalance is confirmed |
| IF one phase current is high while other phases are normal | Phase imbalance, loose connection, winding issue, supply asymmetry, or sensor scaling fault | Do not bypass protection; isolate under LOTO procedure before physical inspection | Measure all phases with an approved instrument and compare against OEM imbalance limits | Escalate to qualified electrical technician |
| IF voltage is below or above the configured approved range | Supply fluctuation, loose terminal, transformer issue, phase loss, wiring problem, or incorrect scaling | Notify electrical maintenance; avoid operating outside OEM voltage limits | Compare all related voltage signals and verify with a calibrated meter | Escalate for phase loss, repeated fluctuation, burning smell, arcing, or protection trip |
| IF temperature is high and the reading is plausible | Overload, blocked ventilation, cooling failure, bearing friction, ambient heat, or lubrication issue | Reduce load or stop according to the approved operating procedure; allow safe cooling | Check airflow, fan, ambient temperature, mechanical friction, and independent temperature measurement | Escalate if temperature continues rising or exceeds OEM shutdown limit |
| IF temperature is implausible or a fallback value is detected | Sensor failure, disconnected probe, PLC scaling error, communication issue, or invalid default value | Mark temperature as invalid; do not use it to declare the machine safe | Inspect sensor wiring, transmitter range, PLC tag mapping, timestamp, and independent measurement | Escalate if machine protection depends on this signal |
| IF Dry_Status indicates an abnormal process state | Dry run, insufficient material/coolant, process interlock, sensor state error, or mapping issue | Follow the machine's approved operating and shutdown procedure | Confirm the actual process state and verify the signal definition with the OEM/controls team | Escalate if dry running can damage the motor, pump, or tooling |
| IF Proxy_Count or Cumulative_Cycles changes unexpectedly | Duplicate events, counter reset, PLC mapping error, gateway duplication, or actual high activity | Do not reset production counters without authorization | Compare PLC counter, event timestamps, increments, and ingestion duplicates | Escalate to controls/data engineering if the mismatch persists |
| IF data is stale, missing, or frozen | Gateway offline, PLC communication failure, network issue, InfluxDB ingestion delay, or sensor fault | Mark telemetry freshness as unknown; do not claim the machine is healthy | Check last timestamp, gateway status, PLC connection, API response, and ingestion logs | Escalate to OT/IT support when freshness exceeds the configured limit |

# 4. Agent Investigation Workflow

1. Read the latest telemetry snapshot and timestamp.

2. Validate freshness, units, ranges, and sensor plausibility.

3. Compare each signal against the approved machine-specific thresholds and OEM limits.

4. Identify the dominant symptom and related signals; do not investigate every signal independently without context.

5. Retrieve relevant sections from service manuals, troubleshooting guides, SOPs, FMEA records, and previous RCA reports.

6. Generate possible causes with evidence from telemetry and retrieved documents. Do not present an unverified cause as confirmed.

7. Recommend inspection or safe next action. Do not directly actuate, reset, bypass protection, or modify machine controls without explicit authorization.

8. Request human confirmation for shutdown, electrical isolation, mechanical intervention, or restart.

9. After maintenance, verify the signal trend, alarm status, and work-order evidence before closing the issue.

# 5. RAG Metadata and Retrieval Tags

machine_id: 00WH9OUI

document_type: machine_repair_troubleshooting

asset_category: industrial_electrical_motor_machine

topics: voltage, current, temperature, sensor_validation, dry_status, cycles, proxy_count

actions: inspect, verify, isolate, escalate, monitor

safety_level: maintenance_authorization_required

source_status: generic_template_pending_oem_validation

# 6. Safety and Authorization Notice

Only trained and authorized personnel may perform electrical, mechanical, or energized-equipment work. Use the plant-approved Lockout/Tagout (LOTO) procedure. Never bypass interlocks, protective relays, alarms, or safety systems. The AI agent may recommend checks and create/escalate an issue, but it must not independently command a shutdown, restart, reset, or physical repair unless the system has an explicitly approved and safety-validated control workflow.

# 7. Required OEM Inputs Before Production Use

Machine make, model, motor rating, rated voltage, rated current, and frequency

Approved normal, warning, and critical thresholds for every signal

OEM temperature and current limits

Phase imbalance and voltage imbalance limits

Exact meaning and valid states of Dry_Status

Sensor ranges, scaling factors, fallback values, and data freshness limits

Approved shutdown, inspection, restart, and LOTO procedures

Responsible role for each escalation type

# 8. Example Agent Output Format

Issue: Average Current Above Configured Limit
Severity: Warning/Critical according to approved threshold
Evidence: Current_AVG, B_Current, R_Current, Y_Current, timestamp, and threshold version
Possible Causes: overload; phase imbalance; mechanical resistance; sensor/scaling issue
Recommended Action: notify authorized maintenance; verify phase currents and mechanical load; do not bypass protection
Verification: calibrated meter comparison, load inspection, and post-maintenance trend check
Confidence: based on available evidence only; root cause not confirmed until physical verification

Document status: Draft template — validate all limits and procedures against the machine OEM documentation and plant safety rules before production deployment.